[image: image1.jpg]s\ &
= ‘,«
. -

o e e e ) e e
2

1
\!
1 o
(il B
w, v
|

e e o od o o o) o

e o e e e od o) o o )

(10 W Wy S S





[image: image4.png]



	[image: image5.jpg]


GaN Microwave Amplifier 
RIK0580K-40T
	[image: image4.png]
	
	
	



	Product Features
	Applications
	[image: image6.jpg]




	• 498.593 ~ 500.593MHz
• High Output Power : 80kW(CW)
• Using GaN on SiC HEMT
• 53% System Efficiency

• High Reliability & Stability, Long-run Operation

• Stable frequency spectrum even at low power
	• Building block for high-power systems
• Accelerlator Light Source

• Industrial Heating and Drying

• Microwave CVD
• Materials processing
• Study of biological phenomena
• Food sterilization / pasteurization
	

	Description

	The RIK0580K-40T is a high-power microwave generator (SSPA) utilizing cutting-edge GaN on SiC HEMT technology, delivering high breakdown voltage and exceptional power efficiency. This product is engineered to meet the stringent and precise requirements of modern particle accelerators and synchrotron light sources.

	Electrical Specifications

	PARAMETER
UNIT
MIN
TYP
MAX
Operating Frequency 

MHz

498.593
-
500.593
Operating Bandwidth

MHz

-
2
-
CW Output Power

W

800
-
80000
System Efficiency
%

53
-
-
Input Power

dBm

-3
-
3
In/Out Return Loss
dB

-
-
-15
Operating Case Temperature
°C
-
-
60

RF Input

-

N-type

Waveguide Output
-

WR1800

Operating Voltage

VAC
380 VAC 3 phase 4wire 50/60 Hz
Monitoring
FWD & RVS Power
System Output and Reflect Power from 10 to 100%
Temperature
Individual Power Amplifier
Current
+50Vdc Output of Power Supply Unit
Maximum mismatch for full power output

6 : 1 VSWR 30sec.
Control Interface
EPICS, Ethernet



	Mechanical Specifications

	PARAMETER
UNIT
VALUE
Dimensions (WⅹDⅹH)

mm

2500 ⅹ 1500 ⅹ 2800
Weight
Kg
1600
Cooling Type
-
Water / Air
Water Cooling Condition
-
 25±1℃, <10bar, >200L/min
Note
Dimension and weight may be subject to change.


	Revision History

	Part Number

Release Date

Version

Description
Data Sheet Status

RIK0580K-40T
May, 2026
0.1
Initial release of datasheet


	


	[image: image2.png]certified

—

K5Q91002018
AS9100D




	Certification
This product is manufactured by a company that is certified for the AS9100D quality management system.



RFHIC Corporation reserves the right to make changes to any products herein or to discontinue any product at any time without notice. While product specifications have been thoroughly examined for reliability, RFHIC Corporation strongly recommends buyers to verify that the information they are using is accurate before ordering. RFHIC Corporation does not assume any liability for the suitability of its products for any particular purpose, and disclaims any and all liability, including without limitation consequential or incidental damages. RFHIC products are not intended for use in life support equipment or application where malfunction of the product can be expected to result in personal injury or death. Buyer uses or sells such products for any such unintended or unauthorized application, buyer shall indemnify, protect, and hold RFHIC Corporation and its directors, officers, stockholders, employees, representatives and distributors harmless against any and all claims arising out of such unauthorized use. All sales inquiries and support should be directed to the local authorized geographic distributor for RFHIC Corporation. For customers in the US, please contact the US sales team through our website at https://rfhic.com/rfhic-us/ . For all other inquiries, please contact our international sales team through our website portal at https://rfhic.com/contact/[image: image3.jpg]210

®) e
b-s conf ~RFL-0946BR >
enalel (25-835we) =i
e
& our
4 10} W
O o FEMALE
{% &
o) ( S
ezl 840
210 200 A




	Korea Facility : +82-31-8069-3000 / www.rfhic.com 
	
	All specifications may change without notice

	US Facility : +1-919-677-8780 / www.rfhic.com/rfhic-us/
	1 / 3
	 Version 0.1



