Product Features	Applications

- 325 MHz (≥ +/-1 MHz)

- Output power: 2kW (CW) Peak 50V

- Built with GaN on SiC HEMT

- 70% System Efficiency

- High Reliability

- Power & Frequency Monitoring Available

- Building Block for High-Power Systems

- Accelerator Light Source

- Plasma Generation

- Industrial Heating and Drying

- Microwave CVD

- Microwave Sintering

- Microwave Chemistry

- Materials Processing

- Study of Biological Phenomena

- Semiconductor Equipment

Description

The RIM032K0-20 is a 2kW Solid-State Power Amplifier (SSPA) engineered with cutting-edge GaN on SiC HEMT technology, delivering exceptional efficiency and high breakdown voltage. This rugged amplifier is meticulously optimized to meet the stringent demands of linear accelerators (Linacs) and particle accelerator RF infrastructures..

Electrical Specifications @ VDS =50V, T=25C, 50 System

| PARAMETER | UNIT | MIN | TYP | MAX |
| --- | --- | --- | --- | --- |
| Operating Frequency | MHz |  | 325 |  |
| Operating Bandwidth | MHz | 324 |  | 326 |
| CW Variable Output Power | W | 25 | - | 2000 |
| Power Gain @Peak Power | dB |  | 72 |  |
| RF Input power | dBm | -10 |  | -8 |
| Efficiency (DC to RF) | % | - | - | 70 |
| Operating Voltage | VDC | - | 50 | - |
| Max. Mismatch @ Max Output Power* | 6: 1 VSWR |  |  |  |

Mechanical Specifications

| PARAMETER | UNIT | VALUE |  |
| --- | --- | --- | --- |
| Dimensions (LⅹWⅹH) | mm | 360 x 200 x 66 |  |
| Weight | kg | 6 |  |
| RF Input Connector | - | SMA, Female |  |
| RF Output Connector |  | 7/16DIN, Female |  |
| DC & GND: DB-8W8 6-Pin Female | - | D-Sub 5W5 |  |
| I/O Connector | - | SMAW200-14C |  |
| Cooling Type | - | Water Cooling Rate |  |
|  |  | Cooling Water Inlet Temperature | 20°C~25°C ((typ.) |
|  |  | Relative humidity below dew point (non-condensing) |  |
|  |  | * De-ionized water shall be used to prevent system damage |  |
| Fluid Inlet/Outlet Size | Inch | 1/4 Tapered Pipe Thread |  |

Note Dimension and weight may be subject to change.

Mechanical drawing

Interface Connector

12 Pin-Control (SMAW200-12C)

Pin No; Description; Pin No; Description. 1; GND; 2; GND. 3; Voltage Monitoring; 4; NC. 5; Forward power Monitoring; 6; Current Monitoring. 7; RF Enable (Low Enable); 8; Temperature Monitoring. 9; INPUT Monitoring; 10; Gain Control (0~5V). 11; Reverse power Monitoring; 12; Phase Control (0~5V). 8 Pin-Power (3005W5SXX99A10X) Pin No; Description; Pin No; Description. A1(131A10049X); 50V; A2(131A10049X); 50V. A3(131A10049X); 50V; A4(131A10049X); GND. A5(131A10049X); GND.

Revision History

| Part Number | Release Date | Version | Description | Data Sheet Status |
| --- | --- | --- | --- | --- |
| RIM032K0-20 | August, 2025 | 0.1 | Initial release of datasheet | Preliminary |

Certification

This product is manufactured by a company that is certified for the AS9100D quality management system.

RFHIC Corporation reserves the right to make changes to any products herein or to discontinue any product at any time without notice. While product specifications have been thoroughly examined for reliability, RFHIC Corporation strongly recommends buyers to verify that the information they are using is accurate before ordering. RFHIC Corporation does not assume any liability for the suitability of its products for any particular purpose, and disclaims any and all liability, including without limitation consequential or incidental damages. RFHIC products are not intended for use in life support equipment or application where malfunction of the product can be expected to result in personal injury or death. Buyer uses or sells such products for any such unintended or unauthorized application, buyer shall indemnify, protect, and hold RFHIC Corporation and its directors, officers, stockholders, employees, representatives and distributors harmless against any and all claims arising out of such unauthorized use. All sales inquiries and support should be directed to the local authorized geographic distributor for RFHIC Corporation. For customers in the US, please contact the US sales team through our website at https://rfhic.com/rfhic-us/ . For all other inquiries, please contact our international sales team through our website portal at https://rfhic.com/contact/